Experiment 6 
 
 
clear; clc; close all;

Fs = 8000;

fc = 1500;

wc = fc / (Fs/2); % Normalized cutoff frequency

N = 50;       	% Filter order

% Window Functions

w_rect  = rectwin(N+1);

w_hamm  = hamming(N+1);

w_hann  = hann(N+1);

w_black = blackman(N+1);

% FIR Filter Design using fir1

b_rect  = fir1(N, wc, w_rect);

b_hamm  = fir1(N, wc, w_hamm);

b_hann  = fir1(N, wc, w_hann);

b_black = fir1(N, wc, w_black);

% Frequency Response Calculation

[H_rect, f] = freqz(b_rect, 1, 1024, Fs);

[H_hamm, ~] = freqz(b_hamm, 1, 1024, Fs);

[H_hann, ~] = freqz(b_hann, 1, 1024, Fs);

[H_black, ~] = freqz(b_black, 1, 1024, Fs);

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf('=== EXPERIMENT 4: FIR WINDOWING TECHNIQUES ===\n\n');

fprintf('Filter Order (N): %d\n', N);

fprintf('Cutoff Frequency: %d Hz\n\n', fc);

fprintf('--- RECTANGULAR WINDOW COEFFICIENTS (First 5) ---\n');

disp(b_rect(1:5));

fprintf('--- HAMMING WINDOW COEFFICIENTS (First 5) ---\n');

disp(b_hamm(1:5));

fprintf('--- HANNING WINDOW COEFFICIENTS (First 5) ---\n');

disp(b_hann(1:5));

fprintf('--- BLACKMAN WINDOW COEFFICIENTS (First 5) ---\n');

disp(b_black(1:5));

% Plotting

figure('Name', 'FIR Windowing Techniques Comparison');

subplot(2,1,1);

plot(f, 20*log10(abs(H_rect)), 'k', 'LineWidth', 1); hold on;

plot(f, 20*log10(abs(H_hamm)), 'b', 'LineWidth', 1.5);

plot(f, 20*log10(abs(H_hann)), 'g', 'LineWidth', 1.5);

plot(f, 20*log10(abs(H_black)), 'r', 'LineWidth', 1.5); grid on;

legend('Rectangular', 'Hamming', 'Hanning', 'Blackman');

title('FIR Low-Pass Filter Magnitude Response (N = 50)');

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-100 10]);

subplot(2,1,2);

plot(f, angle(H_hamm)*180/pi, 'b', 'LineWidth', 1.5); grid on;

title('Linear Phase Response (Hamming Window Example)');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');