EXPERIMENT 2

AIM :

Design and Performance Analysis of Chebyshev Type-I IIR Low-Pass and High-Pass Filters Using MATLAB

SOFTWARE REQUIRED:

MATLAB Simulation

PROGRAM:

Clear;

Clc;

Close all;

Fs = 10000;

Ap = 0.5;

As = 30;

% Low-Pass Filter

Fp_lp = 1500;

Fs_lp = 2000;

Wp_lp = fp_lp/(Fs/2);

Ws_lp = fs_lp/(Fs/2);

[N_lp,Wn_lp] = cheb1ord(wp_lp,ws_lp,Ap,As);

[b_lp,a_lp] = cheby1(N_lp,Ap,Wn_lp,’low’);

% High-Pass Filter

Fp_hp = 2500;

Fs_hp = 1800;

Wp_hp = fp_hp/(Fs/2);

Ws_hp = fs_hp/(Fs/2);

[N_hp,Wn_hp] = cheb1ord(wp_hp,ws_hp,Ap,As);

[b_hp,a_hp] = cheby1(N_hp,Ap,Wn_hp,’high’);

% Display Results

Fprintf(‘CHEBYSHEV TYPE-I FILTERS\n’);

Fprintf(‘LPF Order = %d\n’,N_lp);

Fprintf(‘LPF Cutoff = %.2f Hz\n’,Wn_lp*Fs/2);

Disp(‘LPF Numerator:’); disp(b_lp);

Disp(‘LPF Denominator:’); disp(a_lp);

Fprintf(‘HPF Order = %d\n’,N_hp);

Fprintf(‘HPF Cutoff = %.2f Hz\n’,Wn_hp*Fs/2);

Disp(‘HPF Numerator:’); disp(b_hp);

Disp(‘HPF Denominator:’); disp(a_hp);

% Frequency Response

[H_lp,f] = freqz(b_lp,a_lp,1024,Fs);

[H_hp,~] = freqz(b_hp,a_hp,1024,Fs);

% Plots

Figure;

Subplot(2,2,1);

Plot(f,20*log10(abs(H_lp)+eps),’b’,’LineWidth’,1.5);

Title([‘Chebyshev LPF Magnitude (Order ‘,num2str(N_lp),’)’]);

Xlabel(‘Frequency (Hz)’);

Ylabel(‘Magnitude (dB)’);

Ylim([-60 5]); grid on;

Subplot(2,2,2);

Plot(f,unwrap(angle(H_lp))*180/pi,’r’,’LineWidth’,1.5);

Title(‘Chebyshev LPF Phase Response’);

Xlabel(‘Frequency (Hz)’);

Ylabel(‘Phase (degrees)’);

Grid on;

Subplot(2,2,3);

Plot(f,20*log10(abs(H_hp)+eps),’b’,’LineWidth’,1.5);

Title([‘Chebyshev HPF Magnitude (Order ‘,num2str(N_hp),’)’]);

Xlabel(‘Frequency (Hz)’);

Ylabel(‘Magnitude (dB)’);

Ylim([-60 5]); grid on;

Subplot(2,2,4);

Plot(f,unwrap(angle(H_hp))*180/pi,’r’,’LineWidth’,1.5);

Title(‘Chebyshev HPF Phase Response’);

Xlabel(‘Frequency (Hz)’);

Ylabel(‘Phase (degrees)’);

Grid on;

OUTPUT: