EXPERIMENT 4

AIM:

Design and Comparative Analysis of Butterworth, Chebyshev and Elliptic IIR Filters Using MATLAB

SOFTWARE REQUIRED:

MATLAB Simulation

PROGRAM:

Clc;

Clear;

Close all;

% Input Parameters

Fs = 1000;

Fp = 100;

Fst = 150;

Rp = 1;

Rs = 40;

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

% Chebyshev Type-I Formula

Epsilon = sqrt(10^(Rp/10)-1);

N_formula = acosh(sqrt((10^(Rs/10)-1)/(10^(Rp/10)-1))) …

/ acosh(Fst/Fp);

N_theory = ceil(N_formula);

% Butterworth Filter

[Nb,Wb] = buttord(Wp,Ws,Rp,Rs);

[bB,aB] = butter(Nb,Wb,’low’);

% Chebyshev Type-I Filter

[Nc,Wc] = cheb1ord(Wp,Ws,Rp,Rs);

[bC,aC] = cheby1(Nc,Rp,Wc,’low’);

% Elliptic Filter

[Ne,We] = ellipord(Wp,Ws,Rp,Rs);

[bE,aE] = ellip(Ne,Rp,Rs,We,’low’);

% Display Orders

Fprintf(‘\nIIR FILTER COMPARISON\n’);

Fprintf(‘Butterworth Order = %d\n’,Nb);

Fprintf(‘Chebyshev-I Order = %d\n’,Nc);

Fprintf(‘Elliptic Order = %d\n’,Ne);

Fprintf(‘Chebyshev Formula Order = %d\n’,N_theory);

Fprintf(‘Ripple Factor = %.4f\n’,epsilon);

% Display Coefficients

Disp(‘Butterworth Numerator:’); disp(bB);

Disp(‘Butterworth Denominator:’); disp(aB);

Disp(‘Chebyshev Numerator:’); disp(bC);

Disp(‘Chebyshev Denominator:’); disp(aC);

Disp(‘Elliptic Numerator:’); disp(bE);

Disp(‘Elliptic Denominator:’); disp(aE);

% Frequency Responses

N = 4096;

[HB,f] = freqz(bB,aB,N,Fs);

[HC,~] = freqz(bC,aC,N,Fs);

[HE,~] = freqz(bE,aE,N,Fs);

MB = 20*log10(abs(HB)+eps);

MC = 20*log10(abs(HC)+eps);

ME = 20*log10(abs(HE)+eps);

% Performance

P = f <= Fp;

S = f >= Fst;

RB = max(MB(P))-min(MB(P));

RC = max(MC(P))-min(MC(P));

RE = max(ME(P))-min(ME(P));

SB = -max(MB(S));

SC = -max(MC(S));

SE = -max(ME(S));

Fprintf(‘\nPASSBAND RIPPLE (dB)\n’);

Fprintf(‘Butterworth = %.3f\n’,RB);

Fprintf(‘Chebyshev-I = %.3f\n’,RC);

Fprintf(‘Elliptic = %.3f\n’,RE);

Fprintf(‘\nSTOPBAND ATTENUATION (dB)\n’);

Fprintf(‘Butterworth = %.3f\n’,SB);

Fprintf(‘Chebyshev-I = %.3f\n’,SC);

Fprintf(‘Elliptic = %.3f\n’,SE);

% Magnitude Response

Figure;

Plot(f,MB,’b’,’LineWidth’,1.5);

Hold on;

Plot(f,MC,’r’,’LineWidth’,1.5);

Plot(f,ME,’g’,’LineWidth’,1.5);

Grid on;

Xlabel(‘Frequency (Hz)’);

Ylabel(‘Magnitude (dB)’);

Title(‘Comparison of IIR Filter Magnitude Responses’);

Legend(‘Butterworth’,’Chebyshev-I’,’Elliptic’);

Xlim([0 250]);

Ylim([-80 5]);

Plot([Fp Fp],[-80 5],’k—‘);

Plot([Fst Fst],[-80 5],’k:’);

Hold off;

% Phase Response

Figure;

Plot(f,unwrap(angle(HB))*180/pi,’b’,’LineWidth’,1.2);

Hold on;

Plot(f,unwrap(angle(HC))*180/pi,’r’,’LineWidth’,1.2);

Plot(f,unwrap(angle(HE))*180/pi,’g’,’LineWidth’,1.2);

Grid on;

Xlabel(‘Frequency (Hz)’);

Ylabel(‘Phase (degrees)’);

Title(‘Comparison of IIR Filter Phase Responses’);

Legend(‘Butterworth’,’Chebyshev-I’,’Elliptic’);

Xlim([0 250]);

Hold off;

% Filter Order Comparison

Figure;

Bar([Nb Nc Ne]);

Set(gca,’XTick’,1:3);

Set(gca,’XTickLabel’,{‘Butterworth’,’Chebyshev-I’,’Elliptic’});

Xlabel(‘Filter Type’);

Ylabel(‘Filter Order’);

Title(‘Comparison of IIR Filter Orders’);

Grid on;

OUTPUT: